# Applied Mechanics 1 Formulae

## Rules of Cosine and Sine

c2=a2+b2−2abcosγ

asinα=bsinβ=csinγ

## Linear Motion

v=u+at

s=u+v2t

s=ut+12at2

v2=u2+2a⋅s

where:

- u: Initial velocity

- v: Final velocity

- s: Displacement

- a: Acceleration

- t: Time

## Angular Motion

ω2=ω1∓αt

θ=ω1+ω22t

θ=ω1t∓12αt2

ω22=ω12∓2αθ

where:

- ω1: Initial angular velocity (rad/s)

- ω2: Final angular velocity (rad/s)

- θ Angular displacement (rad)

- α: Angular acceleration (rad/s2)

- t: Time (s)

## Relation Between Linear and Angular Motion

The relationship between linear and angular motion is described by the following equations:

s=rθ (linear displacement s and angular displacement θ).

v=rω (linear velocity v and angular velocity ω),

a=rα (linear acceleration a and angular acceleration α).

## Centre of Gravity

x=∑Moments of Weights∑Weights  y=∑Moments of Weights∑Weights

## Centroid

x=∑xi Ai∑Ai  y=∑yi Ai∑Ai

## Parallel Axis Theorem

To find the moment of inertia about an axis parallel to the centroidal axis:

I=Ic+Ad2

## Radius of Gyration

k=IA (for area) or k=Im (for mass),

where:

- I: Moment of inertia about the axis

- A: Area of the cross-section (for area calculations)

- m: Mass of the body (for mass calculations)

### Rectangle (about its centroidal axis)

- Dimensions: ( b ) (breadth), ( h ) (height)

- Radius of gyration about the centroidal x-axis:

kx=IxA=112bh3bh=h12

### Circle (about its centroidal axis)

- Radius: ( r )

- Radius of gyration:

k=IA=πr44πr2=r2

## Beam Calculations

| Sum of Horizontal Forces | Sum of Vertical Force | Sum of Moments |
| --- | --- | --- |
| ∑Fx=0 | ∑Fy=0 | ∑M=0 |

| Load Type | Shear Diagram Shape | Moment Diagram Shape |
| --- | --- | --- |
| Point Load | Rectangular (constant) | Triangular |
| Uniformly Distributed Load (UDL) | Triangular | Parabolas (second degree) |

## Dynamics

### Linear momentum

Linear momentum=mv

Where:

- Linear momentum is in kg·m/s.

- m is the mass of the object in kilograms.

- v is the velocity of the object in meters per second.

### Angular momentum

Angular momentum=Iω

Where:

- I is the moment of inertia in m4.

- ω is the angular velocity in rad/s.

### Moment of inertia

I=mk2

Where:

- I is the moment of inertia in m4.

- m is the mass in kg.

- k is the radius of gyration in m.

### Turning moment

τ=Iα

Where:

- τ is the torque in Nm.

- I is the moment of inertia in m4.

- α: Angular acceleration in rad/s2.

### Power by Torque

P=τ⋅ω

Where:

P is the power in watts (W),

τ is the torque in newton-meters (Nm), and

ω is the angular velocity in radians per second (rad/s).

### Kinetic Energy of Rotation

Rotational K.E.=12Iω2

Where:

- I is the moment of inertia in m4.

- ω is the angular velocity in radians per second (rad/s).

## Stress and Strain

### Stress

σ=FA

τ=FA

Where:

- σ is the stress (Pa),

- τ is the shearing stress (Pa),

- F is the shearing force (N),

- A is the cross-sectional area (m​2).

### Strain

ε=ΔLL0

Where:

- ε is the strain (unitless),

- ΔL is the change in length,

- L0 is the original length.

### Hooke’s Law

E=σε

Where:

- σ is the stress (Pa).

- ε is the strain (unitless).

- E: Young’s modulus (Pa), a material property (modulus of elasticity).

### Factor of Safety (FOS)

FOS=Breaking StressWorking Stress

## Hydrodynamics

### Volume Flow

v=A⋅C

Where:

v: Volume flow rate, m3/s

A: Cross-sectional area of the flow, m2

C: Mean (average) velocity of the fluid, m/s

### Mass Flow

m=ρ⋅v

Where:

m: mass flow rate, kg/s

ρ: density, kg/m3

v: volume flow, m3/s

### Specific Weight

γ=g⋅ρ

Where:

γ: specific weight, N/m3

g: gravitational acceleration, m/s2

ρ: density, kg/m3

### Continuity Equation

A1⋅C1=A2⋅C2

### Energy Equation

Z1+C122g+P1gρ1=Z2+C222g+P2gρ2

Z1+C122g+P1γ1=Z2+C222g+P2γ2

Each term has units of m, therefore:

- Potential energy Z is known as the elevation head.

- Kinetic energy c22g is known as the velocity head.

- Pressure energy Pγ is known as the pressure head.

Total Head=Elevation Head+Velocity Head+Pressure Head

### Bernoulli’s Equation

P1+12ρC12+ρgh1=P2+12ρC22+ρgh2

Where:

- P1 and P2 are the pressures at points 1 and 2, respectively.

- ρ is the density of the fluid.

- C1 and C2 are the velocities of the fluid at points 1 and 2, respectively.

- g is the acceleration due to gravity.

- h1 and h2 are the heights of the fluid at points 1 and 2, respectively.

Table 1: Centroids of Common Shapes Shape; Area; x; y. A=bh; b/2; h/2. bh2; b/3; h/3. a+bh2; a2+ab+b23a+b; h2a+b3a+b. πr2; r; r. πr22; r; 4r3π. πr24; 4r3π; 4r3π.

## Second Moments of Common Shapes

Table 2: Second moments Shape; Second moment (Ix); Second moment (Iy). Ix=112bh3; Iy=112b3h. Ix=13bh3; Iy=13b3h. Ix=π4r4; Iy=π4r4. Ix=π4r24−r14; Iy=π4r24−r14. Ix=π8r4; Iy=π8r4. Ix=π16r4; Iy=π16r4. Ix=112bh3. Ix=136bh3.